# Robobuoy GPS-Based In-Field Fourier Compass Calibration

Field procedure for generating a GPS-derived Fourier compass correction table using paired heading measurements.

# Purpose

Use GPS start/end bearing measurements to create an 8-point heading correction table that compensates for residual compass installation errors.

# Why Opposite Pairs?

Opposite headings cancel most current and drift effects. Measure 0/180, 45/225, 90/270 and 135/315 degrees.

# Concept Diagram

ICM20948 -> Calibration -> Gyro Fusion -> Fourier Correction -> Final Heading
                               |
                               v
                       GPS Validation

# Current Cancellation

North leg: 0° -> GPS track 8°
South leg: 180° -> GPS track 172°
Pairing removes most current-induced error.

# Calibration Sequence

0° -> 180° -> 45° -> 225° -> 90° -> 270° -> 135° -> 315°

# Procedure

Record home GPS position.

Command heading and speed using TGDIRSPEED.

Wait until heading is stable.

Record start position.

Sail at least 100-150 m.

Record end position.

Calculate GPS bearing from start/end coordinates.

Store heading error.

Repeat for all 8 headings.

Calculate pair corrections and generate Fourier coefficients.

Save interpolation table to the Sub and return home.